EML 4804

Mechatronics Systems

Braydon Fischer

Project 1 Code

22 June 2025

//Password Variable

int password = 40; //Six digit binary password in decimal form for easy password changes, use 40 as the password

//Output Wires

int red = 41; //Variable name for binary pin connected to red LED

int yellow = 43; //Variable name for binary pin connected to yellow LED

int green = 45; //Variable name for binary pin connected to green LED

int trig1 = 53; //Pin connected to the trigger pin of the first ultrasound sensor (US1)

int trig2 = 52; //Pin connected to the trigger pin of the second ultrasound sensor (US2)

int in1 = 22; //Pin connected to the IN1 pin of the L298N controller

int in2 = 23; //Pin connected to the IN2 pin of the L298N controller

int in3 = 24; //Pin connected to the IN3 pin of the L298N controller

int in4 = 25; //Pin connected to the IN4 pin of the L298N controller

int ena = 3;  //Pin connected to the ENA pin of the L298N controller

int enb = 2; //Pin connected to the ENB pin of the L298N controller

//Input wires

int ir1 = 51; //Pin connected to the out pin of the first IR sensor (IRS1)

int ir2 = 50; //Pin connected to the out pin of the second IR sensor (IRS2)

int echo1 = 34; //Pin connected to the echo pin of US1

int echo2 = 32; //Pin connected to the echo pin of US2

//Password input wires

int code0 = 27; //Password input wire for the first digit from the right (lsb) of the password

int code1 = 29; //Password input wire for the second digit from the right of the password

int code2 = 31; //Password input wire for the third digit from the right of the password

int code3 = 33; //Password input wire for the fourth digit from the right of the password

int code4 = 35; //Password input wire for the fifth digit from the right of the password

int code5 = 37; //Password input wire for the sixth digit from the right (msb) of the password

int codedecimal = 0;

//Variables

int IRS1 = 0; //Variable to store the reading from IRS1

int IRS2 = 0; //Variable to store the reading from IRS2

int duration1 = 0; //Variable to store the length of time it took for US1 to receive the echo

int distance1 = 0; //Variable to store the distance an object is from US1, calculated with the duration variable

int duration2 = 0; //Variable to store the length of time it took for US2 to receive the echo

int distance2 = 0; //Variable to store the distance an object is from US2, calculated with the duration variable

void gate1(){     // Function that opens and closes gate 1 and will not close gate 1 if US1 detects that the car is still there

digitalWrite(in1, 1); //Rotates the 1st gate to the open position

analogWrite(ena, 127); //Uses PWM to run the 1st gate motor at 50% duty cycle

delay(300); //Delay of 300 ms to allow gate 1 to open fully

digitalWrite(in1,0); //Stops the 1st gate's rotation

delay(10000); // Need 10 second delay for the open gate

do // do-while loop to prevent the first gate from closing on the car

{

digitalWrite(trig1, 0); //trig pin set to low

delayMicroseconds(2); //2 microsecond delay

digitalWrite(trig1, 1); //trig pin set to high

delayMicroseconds(10); //10 microsecond delay

digitalWrite(trig1, 0); //trig pin set to low after 10 microseconds

duration1 = pulseIn(echo1, HIGH); //stores the amount of time it took for US1 to receive the echo

distance1 = (duration1 * 0.034) / 2.0; // finds how far away the car is by taking the amount of time it took for US1 to receive the echo and multiplying it by the speed of sound in air in centimeters per microseconds (0.034) and dividing that by two, since the sound had to travel there and back

delay(500); //0.5 second delay

}while (distance1 <= 10); //Prevents the first gate from closing on the car

digitalWrite(in2, 1); //Rotates the 1st gate to the closed position

delay(250); //250 ms delay to allow gate 1 to close fully

digitalWrite(in2, 0); //Stops the 1st gate's rotation

delay(2000); // 2 second delay

}

void gate2() {   // Function that opens and closes gate 2 and will not close gate 2 if US2 detects that the car is still there

digitalWrite(in3, 1); //Rotates the 2nd gate to the open position

analogWrite(enb, 127); //Uses PWM to run the 2nd gate motor at 50% duty cycle

delay(250); // 250 ms delay to allow the 2nd gate to rotate fully to the open position

digitalWrite(in3,0); //Stops the 2nd gate motor

delay(10000); // Need 10 seconds delay for the open gate

do // do-while loop to prevent the second gate from closing on the car

{

digitalWrite(trig2, 0); //trig pin set to low

delayMicroseconds(2); //2 microsecond delay

digitalWrite(trig2, 1); //trig pin set to high

delayMicroseconds(10); //10 microsecond delay

digitalWrite(trig2, 0); //trig pin set to low after 10 microseconds

duration2 = pulseIn(echo2, HIGH); //stores the amount of time it took for US2 to receive the echo

distance2 = (duration2 * 0.034) / 2.0; // finds how far away the car is by taking the amount of time it took for US1 to receive the echo and multiplying it by the speed of sound in air in centimeters per microseconds (0.034) and dividing that by two, since the sound had to travel there and back

delay(500); //0.5 second delay

}while (distance2 <=6); // US2 will detect if the car is still there and will not let the gate close until it moves

digitalWrite(in4, 1); //Rotates the 2nd gate to the closed position

delay(250); // 250 ms delay to allow the 2nd gate to rotate fully to the closed position

digitalWrite(in4, 0); //Stops the 2nd gate motor

delay(2000); // 2 second delay

}

void setup() {

//OUTPUT Pins

pinMode(red, OUTPUT); //Initializes the pin for the red LED as an output pin

pinMode(yellow, OUTPUT); //Initializes the pin for the yellow LED as an output pin

pinMode(green, OUTPUT); //Initializes the pin for the greeb LED as an output pin

pinMode(trig1, OUTPUT); //Initializes the pin for US1's trigger pin as an output pin

pinMode(trig2, OUTPUT);  //Initializes the pin for US2's trigger pin as an output pin

pinMode(in1, OUTPUT); //Initializes the pin for the IN1 pin of the L298N as an output pin

pinMode(in2, OUTPUT); //Initializes the pin for the IN2 pin of the L298N as an output pin

pinMode(in3, OUTPUT); //Initializes the pin for the IN3 pin of the L298N as an output pin

pinMode(in4, OUTPUT); //Initializes the pin for the IN4 pin of the L298N as an output pin

pinMode(ena, OUTPUT); //Initializes the pin for the ENA pin of the L298N as an output pin

pinMode(enb, OUTPUT); //Initializes the pin for the ENB pin of the L298N as an output pin

//INPUT Pins

pinMode(ir1, INPUT); //Initializes the pin for the output pin of the IRS1 as an input pin

pinMode(ir2, INPUT); //Initializes the pin for the output pin of the IRS1 as an input pin

pinMode(echo1, INPUT); //Initializes the pin for the echo pin of the US1 as an input pin

pinMode(echo2, INPUT); //Initializes the pin for the echo pin of the US2 as an input pin

}

void loop() {

//Standbye State: IRS1 and IRS2 are on and scanning, red LED on

digitalWrite(red, 1);

IRS1 = digitalRead(ir1);

IRS2 = digitalRead(ir2);

if (IRS1 || IRS2 == 1) //If IRS1 and IRS2 both equal 0, both spots are full, and the gates will not open

{

//First state: US1 on and scanning, red LED on

digitalWrite(trig1, 0); //trig pin set to low

delayMicroseconds(2); //2 microsecond delay

digitalWrite(trig1, 1); //trig pin set to high

delayMicroseconds(10); //10 microsecond delay

digitalWrite(trig1, 0); //trig pin set to low after 10 microseconds

duration1 = pulseIn(echo1, HIGH); //stores the amount of time it took for US1 to receive the echo

distance1 = (duration1 * 0.034) / 2.0; // finds how far away the car is by taking the amount of time it took for US1 to receive the echo and multiplying it by the speed of sound in air in centimeters per microseconds (0.034) and dividing that by two, since the sound had to travel there and back

//Second State: US1 detects car, red and yellow LEDs on, waiting for correct code. Does not happen if both parking spots are full.

if (distance1 <= 10)

{

digitalWrite(yellow, 1);

codedecimal = (digitalRead(code5) * 32) + (digitalRead(code4) * 16) + (digitalRead(code3) * 8) + (digitalRead(code2) * 4) + (digitalRead(code1) * 2) + digitalRead(code0);

delay(2000);

//Third State: Gate 1 opens, only green LED on, delay of 10 seconds before gate 1 closes. Make sure gate does not close on car (US2 must detect car).

if (password == codedecimal) //chec

{

digitalWrite(red, 0); //Turns off the red LED

digitalWrite(yellow, 0); //Turns off the yellow LED

digitalWrite(green, 1); //Turns on the green LED

gate1(); //Executes the gate1 function to open and close the first gate

//Fourth State: Gate 2 opens, only green LED on, delay of 10 seconds before gate 2 closes. Make sure gate does not close on car.

gate2(); //Executes the gate2 function to open and close the second gate

}

}

if(distance1 > 10) //Turns off the yellow LED light only if the car is gone

{

digitalWrite(yellow, 0); //Makes sure that the yellow LED is off

}

digitalWrite(green, 0); //Makes sure that the green LED is off

delay(2000); //2 second delay before restarting the loop

}

}